P-204A Maintenance Work Order

Author: SovAI Sovereign Analytics | CONFIDENTIAL / AIR-GAPPED

## 1. Work Order Information

| Field | Specification |
| --- | --- |
| Work Order ID | WO-P204A-2026-09 |
| Equipment Tag | P-204A (Slurry Transfer Pump) |
| Location | Unit 2 Refining / Pumping Station B |
| Priority Level | High / Corrective Maintenance |
| Target Completion | 2026-09-24 16:00 UTC |
| Work Order Status | Approved for Execution |

## 2. Problem Statement

Primary drive-end mechanical seal leakage and elevated 1X vibration harmonics (4.8 mm/s RMS) were documented during routine round inspection. Slurry seepage past primary barrier threatens bearing contamination.

## 3. Source Evidence

This work order is derived from multi-document historical synthesis across official plant records:

pump_maintenance_history.md (Section 3.2: Mechanical seal replacement history & MTBF)

pump_maintenance_procedure.md (SOP-MNT-204: Slurry pump teardown & seal cartridge replacement)

pump_safety_loto_procedure.md (SOP-SAF-088: Electrical & hydraulic isolation sequence)

daily_shift_log.md (Shift observations on flush fluid pressure drop & vibration spike)

## 4. Current Condition

Equipment is currently isolated from live slurry transfer line. Mechanical seal housing shows minor crystallization residue. Drive-end bearings show temperature stability at 42°C with no catastrophic failure signature.

## 5. Required Maintenance Actions

Execute the following sequential mechanical maintenance procedure in accordance with SOP-MNT-204:

Step 1: Verify zero electrical and hydraulic energy states across primary and bypass lines.

Step 2: Uncouple motor drive shaft and measure baseline axial runout before unbolting casing.

Step 3: Extract degraded mechanical seal cartridge assembly (Part #MS-P204A-HD) and inspect shaft sleeve for scoring.

Step 4: Clean seal chamber bore with approved solvent; install replacement OEM dual-cartridge seal assembly.

Step 5: Re-couple drive shaft, verify alignment within 0.05 mm tolerance, and restore barrier fluid pressure.

## 6. Safety Precautions & LOTO Protocol

⚠️ SAFETY / LOTO DIRECTIVE: CRITICAL SAFETY DIRECTIVE: LOTO Procedure #SOP-SAF-088 must be actively applied before breaking any piping flanges. Mandatory PPE: Chemical-resistant gloves, eye shield, dielectric boots, and H2S/gas monitor. • Apply Lockout/Tagout to breaker MCC-04-P204A and lock out suction/discharge isolation valves V-204-IN and V-204-OUT. • Bleed trapped casing pressure via vent valve V-204-VENT into designated collection sump.

## 7. Required Tools & Materials

| Item | Specification / Part Number | Status |
| --- | --- | --- |
| Seal Cartridge | OEM Dual-Mechanical Cartridge #MS-P204A-HD | Staged in Tool Crib 2 |
| Torque Wrench | 10–150 Nm Calibrated Wrench | Available |
| Dial Indicator | Magnetic Base Dial Indicator (0.01mm resolution) | Available |
| Barrier Fluid | Synthetic Flushing Fluid ISO VG 32 (5 Gallons) | Staged at Job Site |
| Gasket Kit | Viton O-Ring & Flange Gasket Set #GK-204A | Staged in Tool Crib 2 |

## 8. Inspection / Verification

Post-maintenance verification must be performed prior to signing off work order completion:

Verify shaft rotational freedom by manual hand-turn (360° check for binding).

Conduct 30-minute nitrogen pressure decay test at 1.5 bar on seal barrier chamber.

Perform 15-minute operational bump test; verify vibration amplitude remains below 2.0 mm/s RMS.

## 9. Pending Actions

Submit post-repair vibration baseline data to Predictive Maintenance database.

Restock spare mechanical seal cartridge #MS-P204A-HD in warehouse inventory.

Schedule 100-operating-hour re-torque inspection on casing studs.

## 10. Information Gaps & Source Notes

Information Gaps: Not provided in retrieved source records: Exact historical supplier cost breakdown and historical technician permit signoff stamps.

Traceability: All operational limits and procedural steps strictly derived from verified on-premise industrial files.